C-770 Firmware Upgrade Version 84

1. Save the firmware to your hard drive.

2. Extract the file fir77284.bin from C770_v772-84.zip and rename the bin file to firmware.bin

3. Formatted a card in the camera.

Create a folder “firmware” and save the firmware.bin file to the firmware folder.

Save the firmware folder to the root of memory card. Note, you do not have to delete the DCIM folder.

4. Use freshly charged batteries or an AC adapter.

Insert the card with the firmware into the C-770 and turn the camera on in play mode.

5. The LCD display will show the current firmware version.

Then select “YES” (“Yes” will be green) and press “OK”, rewriting of firmware will start automatically.

6. During the rewriting of firmware “EXCHANGING” is displayed on monitor.

It will take a couple of minutes to complete. Please do not operate any key or turn off the power. It might damage the camera and firmware.

7. When the rewriting is done the monitor will show the latest firmware for a spit second and the camera will show the Yes/No screen again. Select NO, press OK and turn the camera off.

Upgrades include:

Version 80 to 81

1) An unusual screen (blue back screen with no warning message) may be displayed on the LCD

for a moment, if the camera is turned on with no card and holding first-release during displaying

startup screen.

2) The alert for a cover open and the icon of the sound recording are overlapped if the card cover is opened while holding first-release in the sound recording mode.

3) Other minute bugs are modified.

# Version 81 to 82

The AF performance is improved as well as the performance of the auto mode.

### Version 82 to 83

1) When the card is full up with the data of panorama shooting, the Exif information of usual still

images are recorded on the card as panorama images, even if the card is formatted or exchanged.

2) When the camera data record zone is exceeded on the xD-Picture Card when the effective block becomes 1000 or less in Zone 0 by aged block failure, a write error causes a PW-OFF.

After restarting the camera, the image disappears and “Card Setup” message is displayed on the LCD monitor. (The management area in the card is deleted.)

3) When playing the MPEG4 movie with sound on the TV monitor with Beep Sound off setting, the sound is not played.

When operating the upper arrow key or lower arrow key, the sound is played. When playing the movie on the camera, this problem doesn’t occur.

4) Color reversing occurs when a camera is connected to specific DVD recorder.

# Version 83 to 84

Sound sync problem has been corrected.

It occurs in the following condition.

- Movie mode: MPEG4 with sound.

- The movie was edited with the camera and played with Quicktime7.